LoRaWAN Infrastructure Setup Manual

Agricultural Sensor • Gateway • Cloud • Evidence • Hyperledger Fabric

Step-by-step installation, configuration, verification and recovery

Project: lorawan-network-server-gateway

Version: Working draft — September 2026

Start with Chapter 1, then follow the setup chapters in order.

# Chapter 1 — System Overview and Setup Order

## 1.1 What the system does

EMU-01 measures agricultural conditions and sends a 46-byte payload over AS923 LoRaWAN. Gateway-01 receives the radio signal and forwards it over the SIM7600 LTE connection. ChirpStack handles LoRaWAN messages, Node-RED saves measurements in PostgreSQL, and Grafana displays them. Separate evidence services preserve and verify original gateway events before eligible records are anchored in Hyperledger Fabric.

Data path: EMU-01 → RAK5146 / Gateway-01 → MQTT / LTE → cloud MQTT → ChirpStack → Node-RED → PostgreSQL → Grafana.

Evidence path: gateway journal → cloud evidence services / SeaweedFS → verifier → Fabric adapter / OpenBao → HRC Fabric.

## 1.2 What each technology does

| Technology | Purpose |
| --- | --- |
| RAK4631 / EMU-01 | Reads agricultural sensors, builds the approved payload and sends a LoRaWAN uplink. |
| ChirpStack Gateway OS / OpenWrt | Runs the Raspberry Pi gateway, networking and gateway services. |
| RAK5146 / Concentratord | Receives AS923 radio packets and passes them to the packet forwarder. |
| SIM7600 LTE | Provides Gateway-01's field Internet connection through wwan0. |
| Mosquitto MQTT | Moves gateway and application events; gateway local broker buffers permitted messages. |
| ChirpStack | Processes LoRaWAN joins, device identities, security, sessions and uplinks. |
| etcd / Patroni / PostgreSQL / TimescaleDB | Coordinates database failover and durably stores measurements and evidence. |
| HAProxy / PgBouncer | Routes clients to the writable PostgreSQL node and pools connections. |
| Valkey / Sentinel | Provides resilient ChirpStack runtime state and cache. |
| Node-RED | Validates/normalizes sensor readings and writes telemetry and Fabric-outbox data. |
| Grafana | Displays read-only measurement histories and system status. |
| SeaweedFS | Retains original raw gateway evidence objects. |
| Gateway evidence services | Collect, match and verify gateway journal, MQTT witness and sensor records. |
| OpenBao / PKI | Protects service identities and provides evidence digest signing. |
| Hyperledger Fabric adapter | Sends eligible exact source payloads to the external HRC ledger and checks finality. |
| Research recorder | Captures, measures and seals formal research test results. |

## 1.3 Equipment and machines

| Device / server | Main task |
| --- | --- |
| EMU-01 | RAK4631 WisBlock agricultural sensor. |
| Gateway-01 | Raspberry Pi, RAK5146, SIM7600, local MQTT, journal and uploader. |
| ULC-01 | Cloud services and normally the only enabled Fabric adapter. |
| ULC-02 | Cloud HA services and disabled Fabric adapter standby. |
| ULC-03 | Cloud HA services, active Node-RED and Grafana. |
| SEC | RUI3 security-test fixture; use only for its prescribed tests. |
| HRC environment | Separate Hyperledger Fabric peers, orderers and chaincode. |

ULC-01/02/03 database roles can change during failover; check the current writable primary before any database operation.

## 1.4 Before starting setup

- Connect the research workstation to the intended management network. Keep the gateway powered off while fitting its concentrator and antennas.

- Prepare Gateway-01, its power supply, Ethernet cable for management, RAK5146 antenna and SIM7600 SIM/antenna.

- Identify the EMU-01 boards and sensors from the assembly chapter. Do not apply power until the wiring and antenna are checked.

- Obtain authorized SSH access to ULC-01/02/03 and the gateway, plus approved service certificates and credentials. Do not copy private keys or passwords into this manual.

- Confirm the required project identities below before provisioning a new device. Do not regenerate an already-registered OTAA identity.

## 1.5 Required project settings

| Setting | Value |
| --- | --- |
| LoRaWAN region | Plain AS923 |
| Gateway EUI | 0016c001f139a1cb |
| EMU-01 DevEUI | ac1f09fffe296d29 |
| EMU-01 JoinEUI | 0000000000000000 |
| Sensor payload | v2 / 46 bytes |
| Activation and device class | OTAA / Class A |
| Gateway production Internet | SIM7600 / wwan0 |
| HRC Fabric endpoint | 10.104.0.7:7051 |
| HRC TLS server name | peer1.hrc.local |

Use the exact frequencies, channel list, data rates and RX parameters from the current gateway, ChirpStack and sensor configurations; do not assume every AS923 installation uses identical defaults.

## 1.6 Setup order

For a new build, follow the chapters in this order. For an existing running system, inspect before rebuilding anything.

- Prepare the Raspberry Pi and install the approved Gateway OS image.

- Fit and configure RAK5146 / Concentratord for the project's AS923 settings.

- Configure SIM7600 LTE, local MQTT and the cloud bridge.

- Prepare ULC-01/02/03: operating system, private networking, Docker and certificates.

- Set up etcd, PostgreSQL / Patroni / TimescaleDB, HAProxy / PgBouncer and Valkey / Sentinel.

- Configure cloud MQTT and two ChirpStack instances.

- Configure SeaweedFS and OpenBao with their own separate storage/consensus state.

- Deploy one active Node-RED writer, PostgreSQL telemetry mapping and Grafana read-only panels.

- Deploy gateway journal/uploader and cloud ingest, collectors and verifier.

- Configure and qualify the sole active Fabric adapter.

- Assemble, flash, provision and test EMU-01.

- Set up the research recorder; run approved tests only after the current readiness gate passes.